## INDICE

1-Tipologia di Esercizi da Sviluppare per gli esami

2-Requisiti Software

3-Linee Guida per sviluppare gli esercizi

4-Descrizioni delle Funzioni Applicom da utilizzare negli esercizi

1-Tipologia di Esercizi

E’ richiesto, ai fini della prova di laboratorio del corso di Informatica Industriale, lo sviluppo di una tipologia di esercizi finalizzati a:

- Testare la comunicazione con la scheda Applicom

- Testare SOLO le funzioni di lettura relativamente al modulo di Ingresso SAIA RIO 16I (indirizzo Profibus DP = 20)

- Sviluppare un applicativo per la suddetta comunicazione, finalizzato allo scambio di bits e di bytes (SOLO IN LETTURA DALLO SLAVE 20)

- Lo sviluppo dei programmi deve essere fatto con l’ambiente Microsoft Visual C++. L’ambiente BorlandC++ dà problemi.

2-Requisiti Software

Per lo sviluppo dei programmi di comunicazione Applicom è richiesto l’uso dell’ambiente Microsoft Visual C++, disponibile anche al centro di calcolo della Facoltà di Ingegneria

3-Linee Guida per sviluppare gli esercizi

Bisogna seguire le seguenti indicazioni:

- Creare un nuovo Workspace (File->New->Workspaces)

- Aggiungere un Progetto dentro il Workspace (ciccare con il pulsante destro del mouse sul nome del workspace e scegliere Add New Project to Workspace). La tipologia di progetto deve essere Win32 Console Application

- Aggiungere i seguenti files dentro al progetto così creato:

  - Applicom.lib

  - Applicom.h

  - Applicom.dll

- Aggiungere il nuovo file in C (NON in C++) che conterrà il vostro programma, dentro il progetto prima creato

- I files Applicom prima citati devono essere copiati dentro la directory del workspace. Il file di esempio accluso contiene tali files.

4-Descrizioni delle Funzioni Applicom da utilizzare negli esercizi

# READPACKIBIT

prototype in C:

void readpackibit(short *, short *, short *, long *, short *, short *)

Syntax in C:

readpackibit(&nchan,&neq,&nb,&adr,tabl,&status)

Parameter		Type

nchan			16-bit integer, Channel number (0-31).

neq			16-bit integer, Equipment number (0-255).

nb			16-bit integer, number of bits to be read (1-2048).

(the maximum number depends on the protocol and target device).

adr			32-bit integer, address of the first bit to be read in the device.

tabl			16-bit integer table receiving read data

status			16-bit integer, interchange status.

Description:

This applicom® standard function can be used to read input bits in a equipment in packed format. The different values are stored in the integer table called 'tabl'. A table item therefore contains a 16-bit chain maximum.  The bits which are not read in the last word that receives the data are in an undetermined state.

The applicom® "transwordbit" function allows you to unpack bit chains.

example in Microsoft C language

Reading of bits 0 to 33 of equipment number 6, from the applicom® interface channel number 1.

#include inimsc.h"	/* Prototype declaration file	*/

short i;	/* For loop counter	*/

short nchan = 1;	/* Channel number	*/

short neq = 6;	/* Equipment number	*/

short nb = 34;	/* Number of variables	*/

short status;	/* Status	*/

long  adr = 0;	/* First variable address	*/

short tabl[3];	/* Table receiving the data	*/

short tablbit[34];	/* Table containing the unpacked values	*/

main()

{

/* Initialize the communication with the applicom® product with "initbus" */

{

do

{

readpackibit(&nchan, &neq, &nb, &adr, tabl, &status);

if (!status)

{

transwordbit(&nb, tabl, tablbit, &status);

for (i=0; i < nb; i++)

printf(" %hd",tablbit[i]);

}

else

printf(" problem on equipment no. %hd \n",neq);

}

while(status == 0);

}

/* execute function "exitbus" before exitting */

exit(0);

}

READPACKIBYTE

prototype in C:

void readpackibyte(short *, short *, short *, long *, char *, short *)

Syntax in C:

readpackibyte(&nchan,&neq,&nb,&adr,tabl,&status)

Parameter		Type

nchan			16-bit integer, Channel number (0-31).

neq			16-bit integer, Equipment number (0-255).

nb			16-bit integer, Number of bytes to be read (1-256).

(the maximum number depends on the protocol and target device).

adr			32-bit integer, Address of the first byte to be read in the device.

tabl			Byte table receiving read data.

status			16-bit integer, interchange status.

Description:

This applicom® standard function can be used to read input bytes in a device. The different values are stored in the byte table called 'tabl'.

example in Microsoft C language

Reading of packed input bytes 0 to 33 in equipment number 6, from the applicom® interface channel number 1.

#include inimsc.h"	/* Prototype declaration file	*/

short i;	/* For loop counter	*/

short nchan = 1;	/* Channel number	*/

short neq = 6;	/* Equipment number	*/

short nb = 34;	/* Number of variables	*/

short status;	/* Interchange status	*/

long adr = 0;	/* First variable address	*/

char tabl[34];	/* Table receiving the data	*/

main()

{

/* Initialize the communication with the applicom® product with "initbus" */

{

do

{

readpackibyte(&nchan, &neq, &nb, &adr, tabl, &status);

if (!status)

{

for (i=0; i < nb; i++)

printf(" %d", tabl[i]);

}

else

printf(" problem on equipment no. %hd \n",neq);

}

while(status == 0);

}

/* execute function "exitbus" before exitting */

exit(0);

}

TRASWORDBIT

prototype in C:

void transwordbit(short*, short*, short *, short *)

prototype in MS pascal:

Syntax in C:

transwordbit(&nb, tabls, tabld, &status)

Parameter	Type

nb		16-bit integer, Number of bits to be unpacked

tabls		16-bit integer table containing the integers to be unpacked.

tabld		16-bit integer table receiving the result (0 or 1).

status		Integer

Description:

This applicom® function allows you to unpack the integer bits in a table. An integer from table "tabls" is unpacked into 16 bits in the table called "tabld".

example in Microsoft C language

Unpack the 16 bits contained in the val integer.

#include "inimsc.h"	/* Prototype declaration file	*/

short i;	/* For loop counter	*/

short nb;	/* Number of bits to be unpacked	*/

short tabls[1];	/* Table containing the data	*/

short tabld[16];	/* Table receiving the result (0 or 1)	*/

short status;	/* Status	*/

short val; 	/* Value to be unpacked	*/

main()

{

/* initialize the communication with the applicom® product with "initbus" */

{

nb =16;

tabls[0] = val;

transwordbit(&nb, tabls, tabld, &status);

if (status == 0)

{

for (i=0; i<nb; i++)

printf( "state of bit %hd = %hd \n ", i, tabld[i]);

}

else

printf("Problem !! status = %hd \n",status);

}

/* execute function "exitbus" before quitting */

exit(0);

}